# Functional Block 3 — Mobile Robot (ASS_Robot)

Product design created by: Arno Gaelens

## Idea Behind the Product

When the wearable sensor detects a fall event, the robot must already be in the correct position to deploy the airbag effectively. A robot that arrives too late or from the wrong side cannot protect the user. The mobile robot solves this by continuously following the person and positioning itself at the rear-left side — the statistically most common fall direction — at all times.

The robot must operate in real indoor environments shared with furniture, walls and other people. It therefore needs to navigate autonomously, avoid obstacles and keep pace with a walking elderly person without being intrusive or creating a trip hazard itself.

The key idea is that the robot acts as a mobile airbag platform: it is not stationary like a bed rail or walker, but follows the person room to room, always ready. The upper section rotates independently so the airbag always faces the person regardless of the robot's driving direction.

## Working Principle

The robot is a differential-drive platform with a rotating upper body (Upper_Swivel). The lower chassis handles locomotion; the upper body carries the airbag module and a LiDAR sensor.

A wearable device worn by the user continuously broadcasts the user's position. The robot subscribes to this position and computes a target point 0.6 m behind and 0.35 m to the left of the person's heading. It drives toward this target using a reactive velocity controller — no pre-planned path, no map required. The upper swivel simultaneously rotates to always face the person, ensuring the airbag is correctly oriented at the moment of deployment.

The system integrates locomotion, sensing, person tracking and obstacle avoidance into one continuous control loop running at 10 Hz.

Although the robot is one integrated unit, it consists of multiple functional subsystems:

- Locomotion — differential drive wheels + passive castor

- Perception — 360° LiDAR

- Person tracking — reactive follower node

- Obstacle avoidance — potential field on raw LiDAR scan

- Upper body orientation — P controller on swivel joint

- Airbag mounting — Upper_Swivel carries the airbag module

### Forces During Operation

The chassis is exposed to drive motor reaction forces, wheel friction against the floor, and reaction forces when the airbag deploys. The castor absorbs minor floor irregularities.

### Temperature Management

Motors and electronics generate low heat during normal walking-pace operation. The airbag module's CO₂ discharge is isolated to the Upper_Swivel and handled by the airbag system's own thermal design.

### Kinematics

The lower chassis performs planar differential-drive motion (translation + rotation). The Upper_Swivel rotates continuously around the vertical axis relative to the chassis, driven by a velocity-controlled motor. No other joints are active.

## Person-Following Algorithm

### Target Point Computation

The robot does not follow the person directly. It targets a point offset from the person:

- 0.6 m behind the person along their heading direction

- 0.35 m to the left of the person

This ensures the robot is always pre-positioned on the most likely fall side.

### Linear Velocity Control

Distance error = distance to target − 0.1 m (desired standoff)

- Error > 0.05 m → drive forward proportionally, capped at 1.2 m/s

- Error < −0.05 m → reverse at up to −0.2 m/s

### Angular Velocity Control

Steering angle = angle to target in robot frame. Blended between person-tracking steering and obstacle repulsion based on proximity to nearest obstacle.

### Obstacle Avoidance (Potential Field)

Each LiDAR ray within ±100° and closer than 1.5 m generates a repulsive angular force:

- Weight = ((obs_dist − r) / obs_dist)²

- Forward-facing obstacles weighted more heavily

- Rays within 50° of person direction ignored if beyond 70% of person distance

- Speed reduction starts at 0.7 m from closest obstacle

- Hard reverse triggered below 0.4 m

### Upper_Swivel Tracking

Swivel error = angle to person − current swivel angle
Swivel velocity = 5.0 × error, capped at ±4 rad/s
This ensures the airbag faces the person at all times regardless of drive direction.

## Material Choices

| Part | Material | Justification |
| --- | --- | --- |
| Chassis frame | [to be filled — Al profile / steel sheet] | Rigidity, machinability |
| Wheel hubs | [to be filled] | — |
| Enclosure panels | [to be filled — ABS / sheet metal] | Impact resistance |
| Swivel bearing | [to be filled] | Continuous rotation under load |
| Fasteners | Stainless steel | Corrosion resistance, durability |

Material section to be completed by mechanical design team from BOM in 0_ProjectInformation/

## Component Selection

### Drive Motors + Wheels

| Field | Value |
| --- | --- |
| Component | Differential drive motors + wheels |
| Model & supplier | [to be filled from BOM] |
| Why this type | Differential drive provides simple, reliable planar navigation with zero turning radius |
| Characteristics | Wheel radius 69.5 mm • Wheel separation 498 mm • Max speed 1.5 m/s |
| Justification | Compact footprint, suitable for indoor use, well-supported in ROS2 |

### LiDAR Sensor

| Field | Value |
| --- | --- |
| Component | 2D LiDAR |
| Model & supplier | [to be filled from BOM] |
| Why this type | Provides 360° obstacle detection for safe navigation in cluttered indoor environments |
| Characteristics | 360° scan • 10 Hz • 10 m range • Mounted on Upper_Swivel |
| Justification | Rotates with upper body so airbag side is always monitored; sufficient range and rate for walking-speed operation |

### Onboard Computer

| Field | Value |
| --- | --- |
| Component | Single-board computer / mini PC |
| Model & supplier | [to be filled] |
| Why this type | Runs ROS2 Jazzy, person-follower node and obstacle avoidance at 10 Hz in real time |
| Characteristics | Ubuntu 24.04 • ROS2 Jazzy • Sufficient CPU for sensor fusion and control loop |
| Justification | ROS2 ecosystem provides well-tested navigation, sensor drivers and communication infrastructure |

### Swivel Motor + Controller

| Field | Value |
| --- | --- |
| Component | Rotary actuator for Upper_Swivel |
| Model & supplier | [to be filled] |
| Why this type | Upper body must continuously rotate to keep airbag facing person |
| Characteristics | Velocity-controlled • ±4 rad/s operating range • ros2_control interface (JointGroupVelocityController) |
| Justification | Velocity control allows smooth P-controller tracking without position overshoots |

### Battery

| Field | Value |
| --- | --- |
| Component | Main drive battery |
| Model & supplier | [to be filled from BOM] |
| Why this type | Powers motors, computer and sensors for a full usage session |
| Characteristics | [voltage, capacity, chemistry — to be filled] |
| Justification | [to be filled] |

## Data Management and Controls

Sensors: LiDAR (360°, 10 Hz, 10 m range), wheel encoders (via odometry), wearable wireless signal (person pose)

Controller: Onboard computer running ROS2 Jazzy

Control process:

- Wearable broadcasts person position → received as /person_pose

- Robot reads own position from wheel odometry → /odom

- LiDAR publishes 360° scan → /scan

- Person-follower node computes rear-left target from person heading

- Potential field computes repulsive steering from nearby obstacles

- Velocity command blended and published → /cmd_vel → drives wheels

- Swivel P-controller computes rotation speed → upper body tracks person

- Loop repeats at 10 Hz

## Accompanying Services

At prototype stage:

- Charging instructions for drive battery

- Basic wheel and joint maintenance

- Software update procedure (ROS2 workspace rebuild)

- Operating instructions for caregiver / operator

Future:

- Remote monitoring dashboard

- Automatic low-battery return to charging station

- Multi-robot coordination

## Implementation Plan

| Phase | Description | Status |
| --- | --- | --- |
| 1 | Gazebo simulation with LiDAR, odometry, cmd_vel, topic bridges | DONE |
| 2 | LiDAR visualization and scan validation | DONE |
| 3 | Mapless Nav2 integration (rolling costmaps, no AMCL) | DONE |
| 4 | Potential field obstacle avoidance + RViz costmap display | DONE |
| 5 | RViz configuration with auto-loaded config file | DONE |
| 6 | Person simulation — click-to-draw path, natural movement | DONE |
| 7 | Reactive person follower with rear-left offset positioning | DONE |
| 8 | Upper_Swivel continuous person tracking via P controller | DONE |
| 9 | Integration with wearable (replace sim pose with real BLE/Wi-Fi signal) | TODO |
| 10 | Integration with airbag trigger (connect follower output to deployment logic) | TODO |
| 11 | Real hardware deployment and tuning | TODO |

## Cost

To be filled from 0_ProjectInformation/ BOM

| Component | Model | Supplier | Qty | Price |
| --- | --- | --- | --- | --- |
| Chassis | — | — | 1 | — |
| Drive motors | — | — | 2 | — |
| Wheels | — | — | 2 | — |
| Castor | — | — | 1 | — |
| LiDAR | — | — | 1 | — |
| Swivel motor | — | — | 1 | — |
| Onboard computer | — | — | 1 | — |
| Battery | — | — | 1 | — |
| Motor drivers | — | — | — | — |
| Fasteners / misc | — | — | — | — |
| TOTAL |  |  |  | — |